| Kit Number: 1234692 KIT SLIM CAN 355ML, 621, 721 1234692-1 KIT, SLIM CAN 355ML, 821 | Date: 11/21/2013 |
| --- | --- |
| Setup Instructions: 1231456 | Rev B |

Subject: Vending 355ml Slim Can                                 		         ECN: 54945

Vendors: All V21 Vendors						         Rev Date: 03/06/14

Purpose: To vend 355ml Slim Can in a V21 machine. (One column only)

|  | Parts List |  |
| --- | --- | --- |
| Quantity | Part Number | Description |
| 621-721: 2 | 1114348-2 | #### Spacer, Mag Foam, .5", 38" |
| 821: 2 | 1114348-4 | #### Spacer, Mag Foam, .5", 45.3" |
| 1 | 1194653 | #### Prod Spacer, Lwr, V21 |
| 1 | 1231456 | #### Product Set-up Inst, 355ml can |

Installation

- Locate backspacer in forward most position. Refer to “Product Setup Guide” decal located on back of inner door for correct placement.

- Install product spacer (p/n 1194653) on right side of column by inserting the tabs in the cutouts.   See Figure #1.

- Install magnet spacers (p/n 1114348-2 for 621/721 machine) on left side of column with the front spacer approximately ½” from the front of the stack and the back spacer shall flush with the front spacer.  See Figure #1.

- Place lower partition clip in forward position.  Refer to “Product Setup Guide” decal located on back of inner door for correct placement.

- Replug power supply and test vend column.

Machine Set-Up:	355ml can

Product Dimensions: Top: 57.4mm; Bottom: 57.4mm;  Length: 155.44mm

A)  Product Depth: 2

B)  Back Spacer:1 (forward most position).

C)  Product Step Location: Forward

D)  Side Shim/ Spacer:1194653

E)  Sold Out Flap: N/A

F)  Sold Out Spacer:N/A

G)  Column Quantity: 40; Column:10 ;Vend Out: 40

H)  Bucket clip: N/A

I)   Magnetic column spacer: P/N:1114348-2 for 621&721, 1114348-4 for 821.   Use 2ea  per column installed side by side on the right side of the column from the back spacer to  ½” front of the column.

Figure 1